CHART 1  ·  Process flow

Ready-to-eat beef jerky  ·  Delta, UT

The customer does not cook it. It contains beef. It is sold shelf-stable, without refrigeration.

Light box = process step    Green box = critical control point    Red box = the lot does not ship

1. Receive inspected beef Invoice and mark of inspection. Measure product temperature.

↓

2. Receive salt, spices, and bags Specification matches. No nitrite. No soy. No wheat.

↓

3. Cold storage — not a CCP Product below 41°F. Prerequisite, stricter than the model’s 45°F.

↓

4. Slice, weigh, marinate, single layer Still below 41°F. Do not stack. Do not dry before the cook.

↓

CCP 1 · COOKING, BEFORE DRYING Internal 145°F for at least 4 minutes Wet bulb at least 125°F for at least 1 hour Relative humidity at least 27% for at least 1 hour Oven sealed for 1 hour or 50% of the cook, whichever is longer Dampers closed within 30 minutes. Come-up 6 hours or less.

↓

CCP 2 · DRYING Dry bulb at least 170°F during drying Water activity 0.85 or less At least six pieces. The highest reading is the result.

↓

5. Cool dry, pack, and label Do not rinse. Only if both CCPs passed. Lot code on the bag.

↓

6. Review the numbers and sign Pre-shipment review under 9 CFR 417.5(c). Then dry storage.

↓

If a CCP limit is missed, the lot is held and is not sold as ready-to-eat beef jerky.

CHART 2  ·  The three limits

From the FSIS beef jerky HACCP model, 2021-0004, and Appendix A

| Point | What is measured | The lot passes if |
| --- | --- | --- |
| Prerequisite | Product temperature | Below 41°F. Not a CCP. |
| CCP 1 Cooking | Internal temperature | 145°F or above for at least 4 minutes |
| CCP 1 Cooking | Wet bulb | At least 125°F for at least 1 hour |
| CCP 1 Cooking | Relative humidity | At least 27% for at least 1 hour |
| CCP 1 Cooking | Sealed oven | Dampers closed within 30 minutes, and kept closed for 1 hour or 50% of the cook, whichever is longer |
| CCP 1 Cooking | Come-up time | From 50°F to 130°F, 6 hours or less |
| CCP 2 Drying | Dry bulb | At least 170°F during drying |
| CCP 2 Drying | Water activity | 0.85 or less. Highest of at least six pieces. |

A blank or the word “yes” does not count. The measured number has to be written. If a CCP number is missing, the lot does not ship. 9 CFR 417.5.

Carne Seca Jesus Canales, LLC · 411 E Main St, Delta, UT 84624 · Limits from the FSIS beef jerky HACCP model (2021-0004) and Appendix A (December 2021).